DSD Question Bank 1

SE EXTC  Sem-III

Subject In-charge : Madhura Shirodkar

| Q. No. | Question | Bloom’s Taxonomy Level | CO Mapped |
| --- | --- | --- | --- |
|  | Module 1 |  |  |
| 1 | Perform the following conversions: - (225.225)10 = ( )2 - (10001.101)2 = ( )10 - (0.365)8 = ( )10 - (0.513)10 = ( )8 - (101101.1101)2 = ( )8 - 725.25)8 = ( )16 | Apply | CO1 |
| 2 | Convert (FFF) in Hex to binary, octal and decimal numbers. | Apply | CO1 |
| 3 | Convert the following signed-magnitude numbers to their decimal equivalent. - 0011 0110 - 1010 1110 | Apply | CO1 |
| 4 | If x = 1101.101 and y = 100.1011, then evaluate - x + y - x – y | Apply | CO1 |
| 5 | If x = 1101.101 and y = 100.1011, then evaluate - x – y using 2’s complement - y – x using 2’s complement | Apply | CO1 |
| 6 | If x = (78)10 and y = (15)10 then evaluate - x – y using 1’s complement - y – x using 1’s complement | Apply | CO1 |
| 7 | Divide (10101011)2 by (101)2 | Apply | CO1 |
| 8 | Represent the decimal number 102.5 in BCD, binary, EX-3 and Gray code form. | Apply | CO1 |
| 9 | Perform using BCD arithmetic - 8124 + 8127 - 8124 – 8127 | Apply | CO1 |
| 10 | Perform multiplication of (11010)2 and (101)2 | Apply | CO1 |
|  | Module 2 |  |  |
| 1 | Define the term ‘Integrated Circuits’. | Remember | CO2 |
| 2 | Give classification of digital ICs based on - Number of components - Types of components | Understand | CO2 |
| 3 | Explain various characteristics of digital ICs. | Understand | CO2 |
| 4 | Explain 2- input TTL NAND gate with open-collector output configuration. | Understand | CO2 |
| 5 | Explain the 2-input standard TTL NAND gate with totem-pole output configuration. | Understand | CO2 |
| 6 | Explain the operation of CMOS inverter. | Understand | CO2 |
| 7 | Explain the operation of 2-input CMOS NAND gate. | Understand | CO2 |
| 8 | Compare TTL and CMOS logic families. | Understand | CO2 |
| 9 | State and prove De Morgan’s theorem. | Apply | CO1 |
| 10 | Convert given SOP F = AB + A’C into - Canonical SOP - Canonical POS | Apply | CO1 |
| 11 | Convert given POS F = (A + B). (B’ +C) into - Canonical POS - Canonical SOP | Apply | CO1 |
| 12 | Simplify the given expression F = X’Y’Z + X’YZ + XY’ using Boolean expression and design using - Basic gates - Only NAND gates - Only NOR gates | Apply | CO1 |
| 13 | Minimize the expression [(AB’ (C +D’) + C’)]’ using Boolean laws. | Apply | CO1 |
| 14 | Simplify the expressions using K-map - F(W,X,Y,Z) = W’X’Y’Z’ + WXY’Z’ + W’X’YZ + WXYZ + WXY’Z + WX’Y’Z’ + WX’YZ - G(A,B,C,D) = ∑m(4,5,6,7,12,13,14) + d(1,9,11,15) - F(P,Q,R,S) = ∑m(0,3,7,8,9,11,12,13) + d(1,4,14,15) - F(W,X,Y,Z) = Πm(1,2,3,8,9,10,11,14)*d(7,15) | Apply | CO1 |
| 15 | Minimize the expressions using Quine-McCluskey method - F(W,X,Y,Z) = W’X’Y’Z’ + WXY’Z’ + W’X’YZ + WXYZ + WXY’Z + WX’Y’Z’ + WX’YZ - G(A,B,C,D) = ∑m(4,5,6,7,12,13,14) + d(1,9,11,15) | Apply | CO1 |
| 16 | Prove that a full adder can be implemented using two half adders and few logic gates. | Apply | CO1 |
| 17 | Describe the principle and working of carry look ahead generator. | Understand | CO1 |
| 18 | Design a two-bit magnitude comparator | Apply | CO1 |
| 19 | Implement a 4-bit BCD adder using IC 7483. Explain its operation by adding 0101 and 0110. | Apply | CO1 |
| 20 | Design a BCD to seven segment decoder. | Apply | CO1 |
| 21 | Design a octal to binary encoder. | Apply | CO1 |
| 22 | Design and Implement the Boolean equation - FA,B,C,D=∑m1,2,3,4,7,8,9,12,14,15 using - Any number 16: 1 MUX - Any number 8:1 MUX - Single 8:1 MUX and few logic gates - Any number 4:1 MUX - Single 4:1 MUX and few logic gates | Apply | CO1 |
| 23 | Implement 1:16 DEMUX using only four 1:4 DEMUX and few logic gates. | Apply | CO1 |