+

-

H(s)

G(s)

K

Rules for Making Root Locus Plots

# The closed loop transfer function of the system shown is

R(s)

C(s)

# T(s) 

KG(s)

1 KG(s)H(s)

So the characteristic equation (c.e.) is

1 KG(s)H(s)  1 K N(s)  0 , or D(s)  K N(s)  0 .

# D(s)

As K changes, so do locations of closed loop poles (i.e., zeros of c.e.). The table below gives rules for sketching the location of these poles for K=0→∞ (i.e., K≥0).

| Rule Name | Description |
| --- | --- |
| Definitions | N(s) - The loop gain is KG(s)H(s) or K . D(s) - N(s), the numerator, is an mth order polynomial; D(s), is nth order. - N(s) has zeros at zi (i=1..m); D(s) has them at pi (i=1..n). - The difference between n and m is q, so q=n-m. (q≥0) |
| Symmetry | The locus is symmetric about real axis (i.e., complex poles appear as conjugate pairs). |
| Number of Branches | There are n branches of the locus, one for each closed loop pole. |
| Starting and Ending Points | The locus starts (K=0) at poles of loop gain, and ends (K→∞) at zeros. Note: this means that there will be q roots that will go to infinity as K→∞. |
| Locus on Real Axis* | The locus exists on real axis to the left of an odd number of poles and zeros. |
| Asymptotes as ¦s¦→∞* | If q>0 there are asymptotes of the root locus that intersect the real axis at n m pi  zi 180  i1 i1 , and radiate out with angles   r , where r=1, 3, 5… q q |
| Break-Away/-In Points on Real Axis | Break-away or –in points of the locus exist where N(s)D’(s)-N’(s)D(s)=0. |
| Angle of Departure from Complex Pole* | m n Angle of departure from pole, pj is depart,pj  180  pj  zi   pj  pi . i1 i1, i j |
| Angle of Arrival at Complex Zero* | m n Angle of arrival at zero, zj, is arrive,zj  180   zj  zi zj  pi  . i1, i j i1 |
| Locus Crosses Imaginary Axis | Use Routh-Hurwitz to determine where the locus crosses the imaginary axis. |
| Given Gain "K," Find Poles | Rewrite c.e. as D(s)+KN(s)=0. Put value of K into equation, and find roots of c.e.. (This may require a computer) |
| Given Pole, Find "K." | Rewrite c.e. as K  D(s) , replace “s” by desired pole location and solve for K. N(s) Note: if “s” is not exactly on locus, K may be complex (small imaginary part). Use real part of K. |

*These rules change to draw complementary root locus (K≤0). See next page for details.

Complementary Root Locus

# To sketch complementary root locus (K≤0), most of the rules are unchanged except for those in table below.

| Rule Name | Description |
| --- | --- |
| Locus on Real Axis | The locus exists on real axis to the right of an odd number of poles and zeros. |
| Asymptotes as ¦s¦→∞ | If q>0 there are asymptotes of the root locus that intersect the real axis at n m pi  zi 180  i1 i1 , and radiate out with angles    p , where p=0, 2, 4… q q |
| Angle of Departure from Complex Pole | m n Angle of departure from pole, pj is depart,pj  pj  zi   pj  pi  . i1 i1, i j |
| Angle of Departure at Complex Zero | m n Angle of arrival at zero, zj, is arrive,zj   zj  zi zj  pi  . i1, i j i1 |