Assignment 3

Q.1) Solve the given  circuit shown in figure & calculate its voltage gain, current gain, input

resistance & output resistance.

Q.2)   Explain with the help of neat diagram the structure of a n- channel FET, and

Its volts amperes characteristics. In what ways it is different from BJT.

Q.3)  Derive the expression for voltage gain, current gain, input resistance &

output resistance for common base amplifier circuit.

Q.4) Determine Z i , Zo & Av for a circuit shown.

Q.5)  Derive the expression for voltage gain, current gain, input resistance &

output resistance for NPN transistor in CE mode with RE unbypassed.

Q.6 ) Draw & explain small signal hybrid pi model of BJT including early effect.

Q.7)  Draw small signal model of JFET & explain significance of each parameter.

Q.8)  Determine the small signal voltage gain & output resistance for a given

Circuit.

Q.9)  Determine the small signal voltage gain, input resistance & output resistance

for a given circuit.

Q.10)  Determine the small signal voltage gain, input resistance & output

resistance  for common source amplifier unbypassed  RS circuit.